### RFA 825:  F-Series DLL Change Guide for Standoff Period Lockout Feature

### I) Technical Changes

1) Select the correct Accounting Mode.

RSI_SETUP_DATA mBasicSetup = new RSI_SETUP_DATA();

mBasicSetup.accountingMode = 0;     //Turn off  Built-In TA mode //mBasicSetup.accountingMode = 1;   //Turn on   Built-In TA mode

2) To enable standoff period, set enableTAStandoff to TRUE (1).

To enable a Password Level 5 menu, set enable_PwL5Menu to TRUE (1).

RSI_EXT_SETUP_DATA tExtSetup = new RSI_EXT_SETUP_DATA();

tExtSetup.enableTAStandoff = TRUE;

tExtSetup.enable_PwL5Menu = TRUE;

3) Set the values for out4C1, out4C2, out4c3, taSO1, taSO2, taSO3, taBC1, taBC2, and taBC3.

RSI_EXT_SETUP_DATA tExtSetup = new RSI_EXT_SETUP_DATA();

tExtSetup.out4C1 = 0x40;

tExtSetup.out4C2 = 0x41;

tExtSetup.out4C3 = 0x42;

tExtSetup.taSO1 = 1;

tExtSetup.taSO2 = 2;

tExtSetup.taSO3 = 3;

tExtSetup.taBC1 = 0x00;  //this will lock out the keypad

tExtSetup.taBC2 = 0x00;  //this will lock out the keypad

tExtSetup.taBC3 = 0x00;  //this will lock out the keypad

TA codes marking the start of a standoff:                                out4C1      out4C2    out4C3

TA Corresponding standoff values in minutes:                    taSO1      taSO2      taSO3

TA codes unavailable to a user during a standoff period:    taBC1      taBC2      taBC3

II Operation

The operation of the standoff functionality is as follows:

The 3 byte sequence OUT4 (out4C1  out4C2  out4C3) is treated as null-terminated.  If an entry is zero, not only does that location not contain an OUT4-code, but any later elements are ignored.  Thus, if out4C1 is zero, there are no OUT4-codes in effect; if out4C2 is zero, then there is only one OUT4-code which is out4C1.

Only if all three bytes are nonzero are there three distinct OUT4-codes.  The 3-byte sequence of BACK-codes (taBC1  taBC2  taBC3) is treated similarly.  The standoff value taSO1 applies to out4C1, the standoff value taSO2 applies to out4C2, etc.

In order to apply lock out to all function keys and Master In/Out, all BACK-codes (taBC1  taBC2  taBC3) must be assigned to zero.

The standoff values represent minutes, and valid values are in the range 1 through 255.

If a particular user enters a punch which generates a TA code that is an OUT4-code, then, if no standoff period is currently in effect for that user, the standoff value corresponding to that TA code is added to the current time and the result is

stored as the standoffEnd for that user.  At any point in time, a standoff period is in effect for that user if the current time is less than or equal to the Standoff End value that was stored for that user.

If a user enters a punch which generates a TA code that is a BACK-code, then no datalog will be generated for that TA code if a standoff period is currently in effect for that user.  If this arises from a Function Key Program, then the message "Invalid Punch" is displayed.  If this arises when the built-in T&A option is in effect, then the associated hand verification will not be accepted.  (Note that in a Function Key Program, the hand verification may already have long since been accepted before it is even known what TA-codes are going to be generated.)

Standoff periods do not have an effect on amnesty punches.

A OUT4-code that occurs during a user's standoff period will not result in any modification to the current standoffEnd for that user.  But a datalog will be generated for it, unless, indeed, the code in question is ALSO listed as a BACK-code.